# Summer Bridge Program – Networks Track Lab Guide

## Installing Windows XP and Server 2003

### Startup

Before starting the installation, you must select the correct machine for the operating system. Each machine has s small green label on the top with a license key for Windows XP Pro. One machine has a label with the key for Windows 2003. Although any machine can be used for Windows XP, you should use the Dell GX620 model for Windows because it is not compatible with Fedora Core 3 Linux.

- For Windows 2003, use the system labeled with its key. Use the GX620 for XP if it is not in use.

- Insert the Windows installation CD and reboot the machine.

- Press Enter to boot from the CD when prompted (you will have only a few seconds).

- Do not select the options to install additional drivers or run a recovery tool.

- When prompted to Setup Windows, press Enter.

### Partitioning and Formatting

At the disk partitioning screen you will be shown a list of the existing disk partitions. You will need to completely clear all existing partitions and create a new one. After partitioning and formatting you will be asked to restart the machine.

- Select and delete each partition.

- Create a new partition and use the default size that it displays (the whole disk).

- Select the new partition and press Enter to install Windows.

- When prompted to format the drive, select NTFS (Quick).

### Installation Options

After some time, you will be prompted to set various options for Windows. You will be asked to set your computer’s name. The Windows 2003 server should be named Pod<#>Server, the XP machine should be Pod<#>XP.  For example, if you are in Pod 1 then your computers should be named “Pod1Server” and “Pod1XP.” You will also be prompted for a password for the local Administrator account, use pod<#>pod<#> (e.g. pod1pod1). Once Windows is loaded, if you are asked to check for updates, activate Windows, register with Microsoft, or anything else, simply select No or Next.

- Regional Settings: No change is required, click Next.

- Name and Organization: Set both to RU

- License Key: Use the license key printed on the label on top of the machine.

- License Mode (2003 only): No change is required, click Next.

- Computer Name: Pod<#>Server or Pod<#>XP

- Administrator Password: pod<#>pod<#>

- Date and Time: You must set the date and time accurately or you may run into problems later when working with Active Directory. Do not forget to set the time zone to Eastern.

- Network Settings (2003 only): Select Typical.

- Workgroup/Domain (2003 only): No change is required, click Next.

- Select No/Next if asked to check for updates, activate, register, etc.

- Create a user (XP only): name = XPuser, password = pod<#>pod<#>.

### Installing Drivers

After Windows is installed, the drivers for the network and graphics card will need to be installed. The “Network Lab Drivers” CD has drivers for each system model used in the lab. The model numbers are printed on the front of the machine around the power button.

- Find the folder on the Drivers CD that matches the model of your machine.

- Double click on each of the files inside of that folder to install the drivers.

- If asked to reboot, say no until all of the drivers are installed.

- If you are told that a version of the driver is already installed, install the driver anyway.

### Configuring Network Settings

Each pod will use IP addresses assigned from the 190.111.<pod #>.0 network. For example, Pod 1’s 2003 server will be 190.111.1.1, XP will be 190.111.1.2. Although it is not setup yet, the Windows 2003 server will be the DNS server for your pod. To make the Windows 2003 server use itself for DNS, you can use the loopback address (127.0.0.1) for its DNS server. Your pod’s router will be the default gateway for the network.

- Open Start->Control Panel and click Switch to classic view.

- Open Network Connections->Local Area Connection

- Set the IP address and network settings.

  - Click Properties.

  - Double-click on Internet Protocol (TCP/IP) at the bottom of the list.

  - IP address:  190.111.<pod #>.1 (Windows 2003) or 190.111.<pod #>.2 (Windows XP).

  - Subnet mask:  255.255.255.0. (do not use the default subnet mask of 255.255.0.0)

  - Default gateway:  190.111.<pod #>.254.

  - DNS server:  127.0.0.1 (Windows 2003) or 190.111.<pod #>.1 (Windows XP).

- Disable the firewall (XP only):

  - Open Start->Control Panel->Security Center->Windows Firewall.

  - Select Off.

## Installing Fedora Core 3

### Startup

Since Linux does not require a license key, there are no machines labeled for it. You may use any machine except the GX620 models, which are not compatible with Fedora Core 3.

- Insert Disc 1 into the CD drive and reboot the machine.

- Press Enter at the boot: prompt to continue with a graphical installation.

- Select Skip when prompted to perform the media test.

- Click next when asked for a language and keyboard mapping.

- Select Custom when asked for the installation type.

### Disk Setup

When you get to the disk partition setup menu, you will be given the choice to let the installer automatically partition for you or to manually partition with Disk Druid. For most people, automatic partitioning should be fine. You will need to delete all existing partitions before installing Fedora. After the disk is partitioned, continue on to Installation Options.

- Select Automatically partition.

- Select Remove all partitions on this system and uncheck Review the partitions created.

- Continue on to the installation options section.

If the automatic partitioning doesn’t work then begin the install again and select the option to partition manually. You will see a list of hard drive devices, such as /dev/sda or /dev/hda, and under each device a list of partition devices, such as /dev/sda1. You might also see a list of volume groups. You will need to delete all of the volume groups and partitions using the Delete button. After they are deleted, you must create three new partitions using the New button. For each partition you will have to select the mount point, the type, and the size.

- Select Manually partition with Disk Druid.

- Delete all volume groups first (e.g. VolGroup00)

- Select the hard drive device (/dev/hda or /dev/sda) and click Delete to remove all partitions on that hard drive.

- Use the New button to create the following partitions

  - mount point = /boot	type = ext3	size = 100

  - mount point = (unset)	type = swap	size = 2000

  - mount point = /	type = ext3	size = (select “Fill to maximum”)

### Installation Options

After the disk has been setup, you will be presented with a series of menus to set the basic options for Linux. When you get to the networking menu, you will be asked for a hostname. The hostname is the computer’s name and domain. You should set the hostname to Pod<#>Linux.pod<#>.edu. For example, if you are pod 2 then your hostname should be “Pod2Linux.pod2.edu.” You will also be prompted for a password for the root account. The root user is the administrative user in Linux. Set the password to pod<#>pod<#> (e.g. pod2pod2):

- Boot Loader: No change required, click Next.

- Network Settings:  Set the hostname to pod<#>Linux.pod<#>.edu.

- Security Settings: Disable the firewall and SELinux (ignore the warning).

- Language: No change required, click Next.

- Time Zone: No change required, click Next.

- Root Password: Set to pod<#>pod<#>.

- Packages: Scroll to the bottom and select the everything option

- Click through the confirmation, welcome, and license agreement screens.

- Date and Time: set the time if needed.

- Display settings: Should be auto-detected but may wish to set the resolution to 1024x786.

- System user (non-administrative user):  username=pod<#>user, password=pod<#>pod<#>

- Additional CDs: None needed, click Next.

### Linux Basics

After booting into Linux, the login screen appears. You should login with a username of root and a password of pod<#>pod<#>. Root is the administrative account on UNIX. If you login as any other user, you will not have permission to make changes to the system.

To access the command prompt, you will need to run a terminal program. You can right-click on the desktop and select Terminal or click on Applications->System Tools->Terminal to open a terminal. At the prompt you can type the name of the command and press Enter to run it. If the command takes an argument, such as a filename, or an option, such as –a, there should be spaces between the name, options, and arguments. The following are basic Linux commands:

- cd <directory>:  change your current location to <directory>. (A directory is the same as a folder.)

- ls: list the files contained within the current directory.

- cp <source files> <destination>: copy files from <source> to <destination>.

- rm <files>: delete <files>.

- gedit <filename>: edit <filename> with a notepad-like text editor.

### Configuring Network Settings

As with the Windows systems, the Linux machine will be assigned an IP address from the 190.111.<pod #>.0 network and will use the Windows 2003 server for DNS and the router as its default gateway. If you are unable to open any applications after changing the network settings, you may have to logout and log back in (as root). Also, some of the computers in the lab have two network interfaces. If you are unable to access the network from Linux, try plugging the network cable into the second network card.

- Open Applications->System Settings->Network.

- Set the IP address, subnet mask, and default gateway.

  - Double-click on eth0 in the list of devices in the Devices tab.

  - Select Statically Assign IP addresses.

  - IP address:  190.111.<pod #>.3.

  - Subnet mask:   255.255.255.0.

  - Default Gateway:   190.111.<pod #>.254.

- Set the DNS Server.

  - Click the DNS tab.

  - Set the primary server to 190.111.<pod #>.1.

  - Verify that the hostname is Pod<#>Linux.pod<#>.edu.

- Click Activate to enable the eth0 interface.
(Note: if you are unable to open a terminal after doing this, click Actions->Logout and re-login as root.)

## Configuring the Router

### Accessing the router

To manage the router, you will first need to connect the router’s console port to the serial port on the back of the Windows XP workstation. This can be done using the flat, light blue console cable. Once connected, the router’s command prompt can be accessed using Hyper Terminal.

On the Windows XP workstation:

- Connect the flat, light blue console cable to the serial port on the back of the machine.

- Connect the other end of the cable to the yellow capped socket on the breakout box.

- Setup a Hyper Terminal session to connect to the router.

  - Open Start->All Programs->Accessories->Communications->Hyper Terminal

  - Name the new connection router.

  - Select COM1 on the Connect To menu.

  - Set Bits per second to 9600 on the COM1 Properties menu.

  - Select File->Save in the Hyper Terminal window to save your setup.

- Press Enter after Hyper Terminal connects. You should see a prompt from the router.

Note: to re-open the router Hyper Terminal session, click Start->All Programs->Accessories->
Communications->Hyper Terminal->Hyper Terminal->router.

### Router command modes

Cisco routers operate in several different modes. Each mode allows only certain commands. The prompt indicates the mode of the router. When you first login to the router, it will be in user mode and the prompt will show Router>. To have access to most commands, you must enter privileged mode. To enter configuration commands, you must be in config mode (note that there are several sub-config modes). You cannot use configuration commands in privileged mode or non-configuration commands in config mode.

Use these commands to switch between modes:

- enable: Enter privileged mode (enable mode). The prompt will be Router#.

- config term: Enter global config mode. The prompt will be Router (config)#.

- exit: exit the last mode entered.

### Initial Setup

The first time you connect to the router, you might be asked to enter setup mode, type no. If you are asked to terminate auto-install, type yes. You will need to set the router’s name, the privileged mode password (enable secret), and the user mode password (console line password).

- enable

- config term

- hostname pod<#>router

- enable secret pod<#>pod<#>

- line con 0

  - password pod<#>pod<#>

  - login

- exit

### Setting IP addresses on the interfaces

The purpose of a router is to forward data between two physically separate networks. Your pod network is connected to one interface (Fa0/0) and the lab backbone network is connected to the other (Fa0/1). When the router receives data packets from a system on your pod’s network that are addressed to a system on a different network, it will check its routing table to see if it knows how to reach that other network. If it does, then it will send the data out on the interface that connects to the other network. Because the router talks to systems on two different networks, each interface must have an IP address from the network that is connected to it.

The IP address assigned to the pod network’s interface is the address your pod’s systems will use as their default gateway. Any time a system wants to send data to a destination that is not on the local network, it will send the data to the default gateway router. The default gateway must then find a path to the destination and forward the data along that route.

From config mode:

- Assign an IP address to the interface that connects to your pod’s network

  - interface fa0/0

  - ip address   190.111.<pod #>.254   255.255.255.0

  - no shutdown

  - exit

- Assign an IP address to the interface that connects to the lab backbone network

  - interface fa0/1

  - ip address   190.111.50.<pod #>   255.255.255.0

  - no shutdown

  - exit

- Exit config mode and show the list of interfaces. Verify that fa0/0 and fa0/1 are up.

  - exit

  - show ip interface brief

### Running RIP

RIP version 2 is the protocol that exchanges routing information between each of the pod routers and the lab backbone router. RIP works by sending advertisements that list all of the networks that can be reached by going through this router. The other pod routers and the lab backbone router see these advertisements and learn routes to each other’s networks. That is, each router will learn that it can send data to any of the other pods by sending the data to that pod’s router first.  So even though the pods are not directly connected to each other, they are all indirectly connected through the same backbone network and can communicate by routing data over the backbone though the router connected to each other pod.

From config mode (config term):

- Start RIP, set the version to 2, and assign both connected networks to RIP.

  - router rip

  - version 2

  - network 190.111.<pod #>.0

  - network 190.111.50.0

  - exit

- After a minute or so, check the routing table for any routes learned from RIP (they will have an R in front of them). Note that if no one else has started RIP, there will not be any RIP routes.

  - exit

  - show ip route

### Creating a Default Route

Normally a router will only forward data to networks that are in its routing table. That is, networks that are directly connected to one of the router’s interfaces, or networks learned from RIP that can be reached by going through a router that is on a directly connected network. When a router connects to the Internet, there are millions of other networks that can be reached. Since the router can’t learn so many routes, it uses a default route (a route to 0.0.0.0) that will match all unknown networks. That is, when the router needs to forward a data packet to a network that it doesn’t know how to reach (such as those on the Internet), it will use the default route set by the administrator to send the packet to another router that will know more about where to forward it.

To be able to access the Internet from your pod, you will have to create a default route on your pod’s router that will forward packets to the lab backbone router (190.111.50.55). The lab backbone router connects the lab network to the campus network and the Internet.

From config mode (config term):

- Create a default route that will send packets to the lab backbone router.

  - ip route   0.0.0.0   0.0.0.0   190.111.50.55

- Exit config mode and verify that the default route is present in the routing table.

  - exit

  - show ip route

### Saving and showing the configuration

After making changes to the running-configuration, you should save it to permanent memory using the command copy run start. You can also list the current running-configuration with show run.

## Required Tasks

### Domain Controller setup

Active Directory domains are used to manage users, computers, and shared resources on a network. The Domain Controller is the Windows server that manages the Active Directory service. Every domain must belong to a Tree, and every tree belongs to a Forest. The tree and forest for your domain will be created automatically when you create your domain. On larger networks, it is possible to have multiple domains in a tree and multiple trees in a forest.

During the installation of Active Directory, you will be asked for the domain name. The domain should be named pod<#>.edu (e.g. pod1.edu).  Also, the installer will warn that a DNS server is not present and ask if it should configure it automatically. Make sure this option is selected. Active Directory will not function correctly without a DNS server.

On the Windows 2003 server:

- Insert the Windows 2003 CD and close the installation menu when it appears.

- Open Start->Manage Your Server->Add or remove a role and select Domain Controller.

- Select Domain Controller for new domain.

- Select the Domain in new forest option.

- Enter pod<#>.edu as the DNS name of the domain.

- Accept POD<#> as the NetBIOS name.

- Accept the default locations for the Database and Log folders.

- Accept the default location for the Shared System Volume.

- Select Install and Configure the DNS server when warned that no DNS server was found.

- Select Permissions compatible with Windows 2000 or Windows 2003.

- Use pod<#>pod<#> for the restore password.

### Configuring DNS

DNS is the system that looks up hostnames, such as Pod1XP.pod1.edu, and translates them into IP addresses (called forward lookups). Networking devices are only capable of using IP addresses but since it is very difficult for most people to remember an IP address, DNS was created to allow the use of hostnames. Anytime a computer is given a hostname, it must perform a DNS lookup to get the IP address.

Because the DNS server was automatically configured by the Active Directory installer you will not need to add the role manually. You will need to create records to map the name of each computer on your pod to its IP address. You will also need to create a Reverse Zone and records that map IP addresses back to hostnames (PTR records). In addition, your DNS server must forward requests for all unknown domains (like those of websites from the Internet) to the lab DNS server to be looked up.

On the Windows 2003 server:

- Open Start->Administrative Tools->DNS and double-click on Pod<#>Server.

- Create a new Reverse Lookup zone.

  - Right-click on Reverse Lookup Zones and select New Zone.

  - Select Primary Zone.

  - Use 190.111.<pod #> as the Network ID.

  - Accept the zone file name by clicking Next.

  - Select Do not allow dynamic updates.

- Add a reverse record for your Windows 2003 server to map its IP address back to its hostname.

  - Right-click on the 190.111.<pod #>.x Subnet reverse zone created above and select New Pointer (PTR).

  - Set the last field in the Host IP number to 1.

  - Set the host name to Pod<#>Server.pod<#>.edu.

- Add a forward and reverse record for the Windows XP workstation.

  - Double-click on Forward Lookup Zones.

  - Right-click on pod<#>.edu and select New Host (A).

  - Set the hostname to Pod<#>XP.

  - Set the IP address to 190.111.<pod #>.2.

  - Select Create associated pointer (PTR) record.

- Add a forward and reverse record the for Linux machine.

  - Right-click on pod<#>.edu and select New Host (A).

  - Set the hostname to Pod<#>Linux.

  - Set the IP address to 190.111.<pod #>.3.

  - Select Create associated pointer (PTR) record.

- Set the DNS server to forward requests for all unknown domains to the network lab server.

  - Select Pod<#>Server in the left panel

  - Double-click forwarders in the right panel.

  - All other DNS domains should be selected in the DNS domains list.

  - Set the Forwarder’s IP address field to 190.111.50.100 and click Add.

### Joining the Active Directory domain

Before it can be managed by Active Directory, the Windows XP workstation must become a member of the domain. When XP attempts to join the domain it will ask for the username and password of a domain administrator. This would be the Administrator account on the Windows 2003 server. After the XP workstation has joined the domain, only users that have been added to Active Directory on the 2003 server will be able to logon to that computer. At this step, the 2003 Administrator is the only Active Directory user.  It is important to notice that at the login screen you will have a choice of logging on to the local computer or to your pod’s domain. Note that Active Directory only works in Windows so the Linux machine will not be able to join the domain.

On the Windows XP workstation:

- Open Start->Control Panel->System.

- Select the Computer Name tab

- Click the Change button

- Select the option to be a member of a domain and set the domain to pod<#>.edu.

- When prompted for a domain administrator login use Administrator and pod<#>pod<#>.

- Restart the computer when asked.

- At the Windows XP login screen, click More Options and select the POD<#> domain.

- Use Administrator and pod<#>pod<#> for the username and password.

### Confirming network connectivity and DNS operation

The ping command is the standard tool for testing connectivity between two machines. It works by sending a series of packets (Echo requests) and waiting for responses (Echo replies). If you do not receive a reply to any of the ping requests then check the network settings on the device that is not responding. If all of the commands succeed from one machine, it is not necessary to repeat them on the others, but you should ping the lab router (190.111.50.55) from each system. By pinging hostnames and using nslookup, you are verifying that the machine is able to contact the DNS server to resolve DNS names.

All of the following commands must be entered on a command prompt. In windows you can access the DOS prompt by opening Start-Run and typing cmd. In Linux, open Applications->System Tools->Terminal. Note that both command prompts support using the up arrow key to access your previous commands. You can also use the right and left arrow keys to edit your previous commands.

From the system of your choice:

- Ping the IP address of each machine.

  - ping 190.111.<pod #>.1

  - ping 190.111.<pod #>.2

  - ping 190.111.<pod #>.3

- Ping the host address of each machine to verify forward DNS records.

  - ping Pod<#>XP.pod<#>.edu

  - ping Pod<#>Server.pod<#>.edu

  - ping Pod<#>Linux.pod<#>.edu

- Use nslookup (a DNS record lookup tool) to verify reverse DNS records.

  - nslookup 190.111.<pod #>.1

  - nslookup 190.111.<pod #>.2

  - nslookup 190.111.<pod #>.3

- Ping both interfaces on your pod’s router and the lab backbone router.

  - ping 190.111.<pod #>.254

  - ping 190.111.50.<pod #>

  - ping 190.111.50.55

- Attempt to open a web page to verify Internet connectivity.

Note that when using the ping command in Linux you will have to press Ctrl-C to stop it from running.

### Sharing a folder

A shared folder is simply a regular folder that is being made available to other computers over the network. A share is identified by its service name, which is similar to a URL. The service name provides a complete path to the shared folder and has a format similar to \\Server-name\Share-name. Shares can be accessed from Windows using My Computer and from Linux with smbclient. Note that, by default, only administrative users will have permission to access the shared folder. Allowing non-administrative users to access the share is done in a later task.

On the Windows 2003 Server:

- Create a folder on the desktop named Shared

- Right-click on the folder and select Properties.

  - Click the Sharing tab and select share this folder.

- Open the folder and create a text file named pod<#>.txt.

  - Click File->New->Text Document.

  - Change the name to pod<#>.txt.

  - Double-click on pod<#>.txt and type a few words in the file.

On the Windows XP workstation:

- Open Start->My Computer.

- Type \\Pod<#>Server\Shared into the Address: bar at the top of the window.

Note that you can also type just \\Pod<#>Server in the Address: bar to see all of the shared folders offered by the server, or you can browse the whole network by clicking My Network Places->Entire Network->Microsoft Windows Network.

### Installing AdminPak

When a Windows XP machine joins a domain, it becomes possible for the domain administrator to manage the domain and all the services on the domain controller (e.g. DNS, DHCP) from the Windows XP workstation. However, Windows XP does not include most of the management plug-ins under Administrative Tools that are required. AdminPak installs the complete set of management plug-ins. AdminPak can be found on the Windows 2003 CD. Once installed, you will use the tools to add a user to Active Directory for each member of your pod. Make sure you are logged in as Administrator on your pod’s domain before installing Adminpak.

On the Windows XP workstation:

- Insert the Windows 2003 CD and close the installation menu when it appears.

- Open Start->My Computer and go to the I386 folder

- Find adminpak.msi, copy it to the desktop, and double-click on it.

- Follow the prompts to complete the AdminPak installation.

- Create a non-administrative user for yourself.

  - Open Start->Administrative Tools->Active Directory Users and Computers.

  - Double-click on pod<#>.edu to expand the list.

  - Right-click on the Users folder and select New->User.

  - Enter your first name in the First Name and User logon name fields.

  - Use Pod<#>Pod<#> for the password (note that the P’s must be uppercase).

  - Uncheck the User must change password at next logon option.

  - Check the Password never expires option.

- Create user accounts for the other members of your pod.

### Remotely managing a computer

It is possible for one computer to connect to another to perform local management tasks. A few of these tasks cannot be used remotely.

On the Windows 2003 server:

- Open Start->Administrative Tools->Computer Management.

- Click Action in the menu bar and select Connect to another computer.

- Select Another computer and enter Pod<#>XP.

- Once connected to the XP workstation, double-click on System Tools.

- Open Device Manager to confirm the connection.

## Elective Tasks

### Configuring mail service

A typical mail server actually runs two standard services, SMTP and POP3. SMTP is the protocol used to send emails to other computers. When an email is sent, the SMTP server forwards the email to the final recipient’s mail server. When a mail server receives an email for a user, it will store that email in the user’s mailbox. POP3 is the protocol used to retrieve email from the mailbox on the server.

Once your mail server is setup, you will have to create a POP3 mailbox for each of the user accounts made for the members of your pod. The passwords for their mailboxes will come from their Active Directory accounts. You can then setup Outlook express on both the Windows 2003 and XP machines to exchange emails.

On the Windows 2003 Server:

- Install the mail server

  - Insert the Windows 2003 CD and close the installation menu when it appears.

  - Open Start->Manage Your Server->Add or remove a role and select Mail server.

  - Enter pod<#>.edu as the E-mail domain name.

- Create a POP3 mailbox for your user account.

  - Open Start->Administrative Tools->POP3.

  - Double-click on Pod<#>Server in the left panel.

  - Right-click on Pod<#>.edu and select New->Mailbox.

  - Enter the username of the account you created for yourself in the Name field.

  - Uncheck Create associated user for this mailbox.

- Create POP3 mailboxes for the other members of your pod.

On both the Windows XP and 2003 machines:

- Setup Outlook Express (make sure you use different email accounts for XP and 2003).

  - Click Start->All Programs->Outlook Express.

  - Set Display Name to your first name.

  - Set the Email address to <your account name>@pod<#>.edu.

  - Use Pod<#>Server.pod<#>.edu for both the incoming and outgoing servers.

  - Set the account password to Pod<#>Pod<#>.

  - Check Logon using Secure Password Authentication.

- Send an email to your teammate on the other Windows machine.

  - Click Create Email and use <teammate’s name>@pod<#>.edu for the “To:” address.

- Check your mailbox on the server for received email.

  - Click the Send/Recv button.

### Enabling Linux FTP and Telnet services

Most of the smaller Internet services in Linux are controlled by a single super server called xinetd. Xinetd has a separate configuration file in the /etc/xinetd.d directory (folder) for each server program that it handles. To enable the FTP and Telnet services, you must edit the files named gssftp and telnet and then restart the xinetd server.

On the Linux machine:

- Open Applications->System Tools->Terminal to access the command prompt.

- Change your current directory to /etc/xinetd.d.

  - cd /etc/xinetd.d

- Edit the gssftp file, enable FTP service, and remove the special authentication option.

  - gedit gssftp

  - Change the disabled = yes line to disabled = no.

  - Remove the –a option from the server_args line.

  - Save and exit.

- Edit the telnet file and enable Telnet service.

  - gedit telnet

  - Change the disabled = yes line to disabled = no.

  - Save and exit.

- Restart the xinetd service.

  - service xinetd restart

Once the services are enabled, you should be able to use the command-line telnet client in Windows XP to remotely access the Linux command prompt and the ftp client to transfer files. Both Telnet and FTP require a login. Most services do not allow the root user to login so you will have to login as the non-administrative user created during the installation (pod<#>user). Note that the backspace key may not work at the telnet login prompt.

On the Windows XP workstation:

- Open Start->Run and type cmd to access the DOS command prompt.

- Open an FTP session to the Linux machine and upload the C:\Config.sys file.

  - ftp Pod<#>Linux.

  - User = pod<#>user, password = pod<#>pod<#>.

  - ls   (list the files in the current directory)

  - put C:\config.sys    (upload the C:\config.sys file)

  - ls    (verify that config.sys was uploaded)

  - quit   (close the ftp connection)

- Open a Telnet session to the Linux machine

  - telnet Pod<#>Linux.

  - Login = pod<#>user, password = pod<#>pod<#>.

  - ls    (list the files in the current directory)

  - exit   (close the telnet connection)

### Accessing a shared folder from Linux

Linux has several ways of accessing a folder shared from a Windows machine, including a command-line tool called smbclient. To access a shared folder with smbclient, you must provide the service name (//server-name/folder-name), a password to access the server, and the name of a user who has access to the share. Once smbclient is connected, it functions very much like an FTP client. It has the commands get (download), put (upload), and ls (list files).

On the Linux machine:

- Open Applications->System Tools->Terminal.

- Access the shared folder on the Windows server as the Administrator user.

  - smbclient   //Pod<#>Server/Shared   pod<#>pod<#>   -U Administrator

  - List the files in the directory with ls.

  - Download the pod<#>.txt file with get pod<#>.txt.

  - Exit smbclient with quit.

### Installing the IIS web server

When first installed, the IIS web server has a default website already pre-configured. The default website’s home folder is C:\Inetpub\wwwroot. This is where the web server will look for web pages for the website. When you access a website and don’t specify a particular web page, the IIS web server will automatically attempt to display the index.htm file, which usually contains the home page for the website (note that all non-Windows web servers use index.html). Although no additional configuration will be required for this task, the web server can be managed from Start->Administrative Tools->Internet Information Services.

On the Windows 2003 server:

- Insert the Windows 2003 CD and close the installation menu when it appears.

- Open Start->Manage Your Server->Add or remove a role and select Application Server.

- Create a simple web page for your pod.

  - Open Start->My Computer->Local Disk (C:)->Inetpub->wwwroot.

  - Click File->New->Text Document.

  - Change the name to index.htm (ignore the warning about the file extension).

  - Right-click on index.htm and type a message (you may use any HTML tags).

On the Windows XP workstation:

- Open Start->Internet Explorer.

- Enter Pod<#>Server.pod<#>.edu. into the address bar.

### Running Terminal Services

The Terminal server allows a remote computer to take complete control of the Windows 2003 server’s desktop. Everything that you would normally see on the server’s screen is sent over the network to another computer. The Terminal server is already pre-configured to allow a Remote Desktop connection and requires no further setup. Note that the Remote Desktops administrative tool on the Windows XP workstation was installed by AdminPak.

On the Windows 2003 server:

- Open Start->Manage Your Server->Add or remove a role and select Terminal server.
(Note that the machine will reboot after the Terminal server is installed.)

On the Windows XP workstation:

- Open Start->Control Panel->Administrative Tools->Remote Desktops.

- Right-click on Remote Desktops in the left panel and select Add New Connection.

- Set the Server Name to Pod<#>Server.

- Double-click on Remote Desktops to expand the list.

- Right-click on Pod<#>Server and select Properties.

- Select the Screen Options tab.

- Select Choose desktop size->800x600.

- Double-click Pod<#>Server to connect to the Windows 2003 desktop and login.

### Creating an Organizational Unit and delegating control to a user

An Organizational Unit (OU) is like a sub-folder inside of Active Directory. It can be used to organize related objects by grouping them together. Any type of object that can be managed by Active Directory (e.g. users, computers, printers) can be placed inside of an OU. An administrator is also able to give control of the OU to a non-administrative user. That user can be allowed to create new users or objects the way an administrator can but only within their OU.

On the Windows 2003 Server:

- Open Start->Administrative Tools->Active Directory Users and Computers

- Create the new Organizational Unit.

  - Right click on the domain’s name, pod<#>.edu, in the left panel.

  - Select New->Organizational Unit.

  - Name the new Organizational Unit Pod<#>.

- Delegate control of the Organizational Unit to a non-administrative user.

  - Right-click on Pod<#> folder in the left panel.

  - Select Delegate Control.

  - Click Add and type the username of one of the accounts previously created for the members of your pod (or you can click Advanced then Find Now to see a list of all users). Once the user appears in the list, click Next.

  - Select at least the first 5 tasks in common tasks list.

To verify that the selected non-administrative user has control over the OU, you will have to logon as that user and attempt to add a user to the Organizational Unit. Because regular users do not have the link to Administrative Tools in their Start menu enabled, you will access Administrative Tools through Control Panel.

On the Windows XP workstation:

- Click Start->Log off to logout then login as the user that controls the Organizational Unit.

- Open Start->Control Panel and click Switch to classic view.

- Open Administrative Tools->Active Directory Users and Computers.

- Right-click on the Pod<#> Organizational Unit in the left panel.

- Select New->User.

  - Use pod<#>user as the First name and Logon name.

  - Use Pod<#>Pod<#> as the password (note that the P’s are uppercase).

  - Uncheck the User must change password at next logon option.

  - Check the Password never expires option.

- Double-click on the Pod<#> OU in the left panel and confirm that the new user was added.

### Configuring DHCP on the Windows 2003 server

DHCP is a widely used protocol that lets a computer learn its network configuration (IP address, Subnet mask, Default Gateway, DNS server and domain) from a central server. This is how the majority of computers that connect to a network or the Internet get their IP address. The DHCP server works by using a specified range of IP addresses from the local network as a scope or pool and then assigning addresses from that range to client computers that send a request for an address.

To make sure that the DHCP server does not try to give out one of the addresses that are already being used (1, 2, 3, and 254), the scope’s range should not include every address in the network (1-254). The scope can also be configured to send other network settings, such as the DNS server and default gateway. To test the DHCP server, the Windows XP workstation should be set to obtain its IP address automatically.

Note that when the DHCP server is fully functional, there should be a small green arrow next to the name of the server. If there isn’t then you will need to authorize the server. To do that, double-click on the server’s name in the left panel. There should be a message giving you the option to authorize the server..

On the Windows 2003 server:

- Open Start->Manage Your Server->Add or remove a role and select DHCP server.

- Set the scope name to Pod<#>Scope.

- Use 190.111.<pod #>.10 as the starting IP address and 190.111.<pod #>.50 as the ending address.
Set the subnet mask to 255.255.255.0 if it is not already set.

- No addresses need to be excluded from range used by the scope, click Next.

- No change is needed to the default address lease time, click Next.

- Select yes to configure DHCP options.

  - Set the Default Gateway to 190.111.<pod #>.254 and click Add.

  - Set the parent domain to pod<#>.edu. 
Set the server IP address to 190.111.<pod #>.1 and click Add.

  - No WINS server is needed, click Next.

- Activate the Scope.

On the XP workstation:

- Open Start->Control Panel->Network Connections->Local Area Connection.

- Click Properties.

- Double-click on Internet Protocol (TCP/IP) at the bottom of the list.

- Select Obtain IP address automatically.

- Select Obtain DNS server address automatically.

- Click OK to close the property Window and go to the Support tab on the Local Area Connection Status window.

- Click the Repair button to trigger a DHCP request. The status panel should display the new IP address and that it was assigned by DHCP.

### Configuring DHCP on the router

DHCP on the router is very similar to DHCP on Windows 2003 except it uses a pool instead of a scope. The router also assumes that every address on the pod’s network can be used for the pool so you will have to specifically exclude the range of addresses that it should not use.

On the router in config mode (config term):

- Exclude the ranges 1-50 and 100-254 from the DHCP pool.

  - ip dhcp excluded-address   190.111.<pod #>.1   190.111.<pod #>.50

  - ip dhcp excluded-address   190.111.<pod #>.100   190.111.<pod #>.254

- Create the DHCP pool and set the DHCP options.

  - ip dhcp pool   Pod<#>Pool

  - network    190.111.<pod #>.0    255.255.255.0

  - default-router   190.111.<pod #>.254

  - dns-server   190.111.<pod #>.1

  - domain-name   pod<#>.edu

  - exit

Because DHCP is already running on the Windows 2003 server, it will have to be temporarily suspended before testing DHCP on the router. You can disable the Windows 2003 DHCP service by right-clicking on the scope in the DHCP management component and selecting Deactivate. To re-enable DHCP, select Activate. DHCP service on the router can also be suspended by entering the command no service dhcp in config mode and can be re-enabled with service dhcp.

On the Windows 2003 server:

- Open Start->Administrative Tools->DHCP.

- Double-click on the server’s name, Pod<#>Server, in the left panel to show the list of scopes.

- Right-click on the scope named Pod<#>Scope.

- Select Deactivate.

Since the Windows XP machine was already assigned an IP address by the Windows 2003 DHCP service, it will not automatically request a new one now that the router is providing DHCP service. You will have to click Repair on the network interface to force Windows XP to request a new IP address from the router’s DHCP service.

On the XP workstation:

- Open Start->Control Panel->Network Connections->Local Area Connection.

- Select the Support tab.

- Click the Repair button to trigger a DHCP request.

- Click Details and verify that the router (190.111.<pod #>.254) is the DHCP server that assigned the new IP address.

### Attaching a networked printer

To allow a standard printer to connect directly to the network, we must use a small device called a print server. The print server connects to the network and waits for other systems to send printer data to it. When data is sent, the print server forwards the data to the attached printer. Before the printer server can communicate over a network, it has to acquire an IP address, so there must be a DHCP server running on your pod’s network. Once the print server is configured, the Windows 2003 server must be configured to send printer data to the print server’s IP address. It also needs to know the model of the printer. Most of the printers in the lab are HP Deskjet 970Cxi models. (There are also two HP Deskjet 3820 models for pods 5-7 that require a separate driver disk).

On the print server:

- Select any print server and connect it to the jack on the breakout labeled with your pod’s number.

- Clear any previous configuration from the print server.

  - Disconnect the power cable from the back of the print server.

  - Hold down the small red test button.

  - Reconnect the power cable and release the test button.

  - Wait one minute for the print server to establish the physical connection to the network.

- Have the printer server obtain an IP address by holding in the test button for approximately 5 seconds. When it prints its configuration, find the DHCP assigned IP address on the second page.

On Windows 2003 server:

- Open Start->Control Panel->Printers and Faxes.

- Right-click in an empty part of the window and select Add Printer.

- Select Local Printer attached to this computer and uncheck Automatically detect.

- Select Create a new port and set the type to Standard TCP/IP Port.

- Enter the IP address from the information page into the Printer Name or IP Address field.

- Select HP for the manufacturer and HP Deskjet 970Cxi for the printer.

- Select Yes to using this as the default printer.

- Select Share this printer and set the Share Name to Printer.

- No location or description information is needed, click Next.

- Select Yes to print a test page.

### Restricting permissions of a shared folder and auditing

Security permissions can be set on a folder (or any other object) to control what tasks a user can and cannot perform on that folder. Shared folders have two types of permission settings, Share Permissions and Access Control Lists. The Share Permissions are set by clicking on the Permissions button in the Sharing tab of the folder’s properties. Access lists are managed in the Security tab. It is important to realize that both sets of permissions are used by the server to determine what each user will be permitted to do. The server will always pick the most restrictive interpretation of the settings. For example, if the share permissions grant write access to everyone but the access list denies write access for some users, then the more restrictive deny entry will take precedence over the less restrictive share permissions.

By default, the shared folder created earlier has share permissions that allow all users to open and read files from the folder but write to or delete. It also has a more restrictive access control list that only permits administrators to access the folder. So by default, the shared folder cannot even be opened by non-administrative users. To give all users the ability to read, write and delete files, the share permissions can be set to give Full Control permissions to Everyone. Also, a new Access Control List entry must be added for the Everyone group that also grants Full Control permission. Once this is done, all non-administrative users should have permission to do anything in the shared folder.

To deny write and delete permission for one specific user, you have to create an access list entry that specifically denies write and delete permission to that user. Delete permission is not included in the basic access list menu so an advanced access list entry must be used. Note that creating an access list for the user that doesn’t deny permission but doesn’t allow it either is not the same as actually denying permission.

Auditing is a system that allows administrators to track and log when a user makes an attempt (successful or failed) to access system resources or perform certain tasks. In this case, it will be used to monitor each time a specific user is denied permission to write or delete files in the shared folder. Auditing on a folder is configured in the Security tab and works almost exactly like file permissions. Before you can create auditing rules on a folder, you must enable Object Access auditing in the Domain Controller Security Policy (any auditing rules created before enabling the policy will not work). Once auditing has been established, any failed attempts to write or delete files in the shared folder will be logged to the Security log of the Event Viewer. To test auditing on a shared folder, you must login as the restricted user on the Windows XP workstation and attempt to delete or write to the file. The permissions should cause the attempt to fail and the auditing rule should log the failed attempt.

On the Windows 2003 server

- Right-click on the folder named Shared and select Properties.

- Set the share permissions to grant Full Control to all users

  - Select the Sharing tab and click on the Permissions button.

  - Select the Everyone entry in the user and groups list.

  - Select the Allow option for Full Control permission in the permissions panel.

- Create an Access Control List entry that grants Full Control to all users.

  - Select the Security tab.

  - Click the Add button below users and groups and type Everyone.

  - Select Everyone and check the Allow option for Full Control permission in the list.

- Deny write permission to one of the accounts created for the members of your pod.

  - Click the Add button and type in the username of a non-administrative user.

  - Select the added user and uncheck all Allow permissions in the permissions list.

  - Check the Deny option for Write permission.

  - Click the Advanced button.

  - Select the Deny entry associated with the added user and click Edit.

  - In the permissions list, check the Deny options for Delete and Delete Files and Subfolders.
(Note that you should see several Deny options already checked for write permissions.)

  - Click OK to return to the Advanced Settings window.

- Enable auditing for object (file and folder) access in the domain controller policies.

  - Open Start->Administrative Tools->Domain Controller Security Policy.

  - Double-click Local Policies->Audit Policy->Audit object access.

  - Select Define this policy and Failed.

- Create an auditing entry to track failed access attempts made by the user.

  - Select the Auditing tab in the Advanced Settings window.

  - Click Add and type the username of the selected account.

  - In the object access panel, select the Failed option for the six write or delete access methods starting at Create Files/Write Data and ending at Delete.

  - Click Apply and OK on all of the windows associated with the folder’s properties.

- Open the Event Viewer Security log, clear all exiting log entries, and watch for a Failed entry.

  - Open Start->Administrative Tools->Event Viewer.

  - Select Security in the left panel.

  - Right-click on Security and select Clear all events. Click No when asked to save to a file.

  - When an access is attempted, right-click on the right panel and select refresh to see it.

On the Windows XP workstation:

- Click Start->Log off to logout then login as the user who has the restricted permissions.

- Open Start->My Computer and type \\Pod<#>Server\Shared into the address bar.

- Right-click on the pod<#>.txt file and attempt to delete it (you should see permission denied).

- Check the Event Viewer Security log on the Windows 2003 server for a failed access event.

### Creating a router Access Control List

As a router is forwarding data packets from one network to another, it normally forwards every packet that comes in on every interface. An access control list tells the router to only forward certain packets and not others. An access list consists of a series of entries that either permit or deny a packet. Depending on the type of list, entries can match a packet’s source or destination IP address, source or destination port, and protocol. Standard lists (1-99) only match the source IP address. Extended lists (100-199) can match anything.

Access lists also require a wildcard mask for each IP address used. A wildcard mask can be thought of as an inverse subnet mask. If you want an entry to match an entire network that has a mask of 255.255.255.0 then the wildcard mask would be 0.0.0.255. To match a single IP address, you would use a wildcard mask of 0.0.0.0. To have the entry match any possible address, you can use the keyword any in place of the address/wildcard-mask combination.

After the list is created, it must be applied to an interface and set to match either inbound or outbound packets. Once in place, the router will compare every packet to each entry in the access list. If no entry matches the packet, there is an implicit deny statement at the end of the list. That is, all unmatched packets are automatically denied. Note that packets are only matched if they must go through the router to reach their destination. That is, packets sent between the systems within your pod’s network will not go through the router and so will not be filtered by the access list.

You will have to pick a single Linux machine from one of your neighboring pods and create an access entry to block all packets from this machine. The list will also block telnet access from everywhere. Once you have selected the pod and created the list, you can test the list from that pod. If the list is successful, the Linux machine will not be able to ping anything in you pod and all attempts to telnet to your pod should fail. Everything else, should work as normal.

On the router in config mode (config term):

- Create an access list entry that will block telnet access from any host outside of the network.

  - access-list 100 deny tcp   any   190.111.<pod #>.0   0.0.0.255   eq telnet

- Create an entry to block any ip traffic from the Linux machine in a different pod to this pod.

  - access-list 100 deny ip 190.111.<other pod #>.3   0.0.0.0  190.111.<pod #>.0    0.0.0.255

- Create an entry to match all other packets and allow them.

  - access-list 100 permit ip any any

- Apply the list to the fa0/0 interface to match packets going out to the pod’s network.

  - interface fa0/0

  - ip access-group 100 out

  - exit

From the pod with the blocked Linux machine:

- Verify that you can ping your pod from the Windows XP workstation but cannot telnet.

  - Open Start->Run and type cmd.

  - ping 190.111.<your pod #>.3

  - telnet 190.111.<your pod #>.3

- Verify that you cannot ping systems in your pod from the Linux machine.

  - Run Applications->System Tools->Terminal.

  - ping 190.111.<your pod #>.1

  - ping 190.111.<your pod #>.3

### Sharing DHCP service with a second pod

It is possible to use a single DHCP server to assign IP addresses to multiple networks. Since each pod is a separate network with a different set of IP addresses, you must create a second scope/pool for the second pod. The client from the second pod must be assigned the correct IP address, default gateway, and DNS server for its network, which will be different from your own pod’s network.

On the 1st pod’s Windows 2003 server:

- Open Start->Administrative Tools->DHCP.

- Right-click on Pod<#>Server in the left panel and select New Scope.

- Set the scope name to Pod<2nd pod #>Scope.

- Use 190.111.<2nd pod #>.10 as the starting address and 190.111.<2nd pod #>.50 as the ending address.
Set the subnet mask to 255.255.255.0 if it is not already set.

- No addresses need to be excluded from range used by the scope, click Next.

- No change is needed to the default address lease time, click Next.

- Select yes to configure DHCP options.

  - Set the Default Gateway to 190.111.<2nd pod #>.254 and click Add.

  - Set the parent domain to pod<2nd pod #>.edu. 
Set the server IP address to 190.111.<2nd pod #>.1 and click Add.

  - No WINS server is needed, click Next.

- Activate the Scope.

For a DHCP request from one pod to reach the DHCP server in another, it must go through the routers of both pods. DHCP requests are sent as broadcast packets (packets addressed to everyone on the local network) and a router will not normally forward a broadcast packet outside of its own network. To permit DHCP sharing, the router on the second pod must be configured with a helper address that points to the first pod’s DHCP server. The second pod’s router will listen for DHCP requests on its network and forward them to the DHCP server’s address on the first pod using a regular (non-broadcast) packet.

On the 2nd pod’s router in config mode (config term):

- Set the IP helper address to the first pod’s Windows 2003 server.

  - interface fa0/0

  - ip helper address 190.111.<1st pod #>.1

  - exit

- Suspend DHCP service on the router (if it is running).

  - no service dhcp

To ensure that DHCP service is being provided by the first pod, you must suspend all DHCP services on the second pod. That is, deactivate the pod’s scope in Windows 2003 and disable the DHCP service on the router. As before, you must also force the Windows XP workstation to request a new IP address using the repair button and click the details button to verify that it received an address from the first pod’s server.

On the 2nd pod’s Windows 2003 server:

- Open Start->Administrative Tools->DHCP.

- Double-click on the server’s name, Pod<2nd pod #>Server, in the left panel to show the list of scopes.

- Right-click on the scope named Pod<2nd pod #>Scope.

- Select Deactivate.

On the 2nd pod’s Windows XP workstation:

- Open Start->Control Panel->Network Connections->Local Area Connection.

- Select the Support tab and click the Repair button to trigger a DHCP request.

- Click Details and verify that the Windows 2003 DHCP server on the 1st pod assigned the address.

### Connecting two Active Directory domains using a trust relationship

To connect two domains that are in separate forests, you must establish a trust relationship. A trust will allow the systems in each domain to login to the accounts and access the resources of the other domain. It also allows both domains to be managed from the Administrative Tools of a single system. Once a trust has been established, you should be able to login to an account on the other domain.

It is very important that both Windows servers have as close to the same time as possible. If their clocks are not synchronized, you will get an error when attempting to connect the domains. It is best to synchronize both clocks with an Internet time server, so your Internet connection should be up before attempting this task. If you are unable to use an external time server, it is possible to synchronize the clocks by hand.

Because Active Directory requires DNS to manage the domains, each domain must be able to access the DNS records of the other domain. Your pod’s DNS server does not have any information about the other domain, nor does the lab server. So you must forward DNS requests regarding the other domain to that other domain’s DNS server.

On your pod’s Windows 2003 server:

- Synchronize the clocks of the Windows 2003 servers in both pods.

  - Right-click on the clock in the taskbar and select Adjust Date/Time.

  - Select the Time Zone tab and verify that the zone is set to Eastern.

  - Select the Internet Time tab and click Update Now.

  - Select the Date and Time tab and verify that both servers have the same date and time.

- Forward DNS requests for the 2nd pod’s domain to that pod’s DNS server.

  - Open Start->Administrative Tools->DNS

  - Select Pod<#>Server in the left panel

  - Double-click forwarders in the right panel.

  - Click New to add a new DNS domain and enter the 2nd pod’s domain name.

  - Select the newly added domain.

  - Set the Forwarder’s IP address field to 190.111.<2nd pod #>.1 and click Add.

- Create a two-way trust between your pod’s domain and the 2nd pod.

  - Open Start->Administrative Tools->Active Directory Domains and Trusts.

  - Right-click on pod<your pod #>.edu in the left panel and select Properties.

  - Select the Trusts tab and click New Trust.

  - Enter pod<2nd pod #>.edu.

  - Select Two-way trust.

  - Click through the confirmations.

- Repeat these steps on the 2nd pod’s Windows 2003 server using the 1st pod’s domain and DNS server in the trusts and forwarders setup screens.

On the Windows XP workstation in both pods:

- Log out by clicking Start-Log off.

- Enter the username of an account in the other pod’s domain at the Login screen.

- Click More Options and select the other pod’s domain from the list.