- CCNA: Introduction to Networks

Retake - Skills Assessment – Student Training (Answer Key)

- Name:                                                                   .

- Topology

- Assessment Objectives

Part 1: Develop the IPv4 Address Scheme (15 points, 0 minutes)

Part 2: Configure Device IPv4 and Security Settings (65 points, 20 minutes)

Part 3: Test and Verify IPv4 End-to-End Connectivity (10 points, 10 minutes)

Part 4: Use the IOS CLI to Gather Device Information (10 points, 5 minutes)

- Scenario

In this Skills Assessment (SA) you will configure the devices in a small network. You must configure a router, switch and PCs to support both IPv4 and IPv6 connectivity. You will configure security, including SSH, on the router. You will test and document the network using common CLI commands. Finally, you will save the router configuration to a TFTP server.

- Required Resources

- 1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

- 1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

- 2 PCs (Windows 7, Vista, or XP with terminal emulation program, such as Tera Term)

- Console cable to configure the Cisco IOS devices via the console ports

- Ethernet cables as shown in the topology

- Develop the IPv4 Addressing Scheme

Ref labs:	9.2.1.3 Lab - Designing and Implementing a Subnetted IPv4 Addressing Scheme

9.2.1.4 Lab - Designing and Implementing a VLSM Addressing Scheme

Total points: 15

Time: 20 minutes

Given an IP address and mask of 192.168.25.0/24 (address/mask), design an IP addressing scheme that satisfies the following requirements:

| Subnet | Number of Hosts |
| --- | --- |
| Subnet A | 2 |
| Subnet B | 29 |

| Subnet A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification |  | Student Input |  | Points |
| Number of bits in the subnet |  | 6 |  | (5 points) |
| IP mask (binary) | 11111111.11111111.11111111.11111100 |  |  |  |
| New IP mask (decimal) |  | 255.255.255.252 |  |  |
| Maximum Number of usable hosts per subnet |  | 2 |  |  |
| IP Subnet |  | 192.168.25.32 |  |  |
| First IP Host address |  | 192.168.25.33 |  |  |
| Last IP Host address |  | 192.168.25.34 |  |  |

| Subnet B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification |  | Student Input |  | Points |
| Number of bits in the subnet |  | 3 |  | (5 points) |
| IP mask (binary) | 11111111.11111111.11111111.11100000 |  |  |  |
| New IP mask (decimal) |  | 255.255.255.224 |  |  |
| Number of usable hosts per subnet |  | 30 |  |  |
| IP Subnet |  | 192.168.25.0 |  |  |
| First IP Host address |  | 192.168.25.1 |  |  |
| Last IP Host address |  | 192.168.25.30 |  |  |

| Device | IP address | Mask | Gateway | Points |
| --- | --- | --- | --- | --- |
| PC-A | 192.168.25.33 | 255.255.255.252 | 192.168.25.34 | (5 points) |
| Router1-G0/0 | 192.168.25.34 | 255.255.255.252 | N/A |  |
| Router1-G0/1 | 192.168.25.30 | 255.255.255.224 | N/A |  |
| S1-VLAN1 | 192.168.25.29 | 255.255.255.224 | N/A |  |
| PC-B | 192.168.25.1 | 255.255.255.224 | 192.168.25.30 |  |

Instructor Sign-off Part 1: ______________________

Points: _________ of 15

- Configure Device IPv4 and Security Settings

Ref lab: 11.2.4.6 Lab - Securing Network Devices

Total points: 30

Time: 20 minutes

  - Configure host computers.

After configuring each host computer, record the host network settings with the ipconfig /all command.

| PC-A Network Configuration |  | Points |
| --- | --- | --- |
| Description | You don’t need to configure this. | (2 points) |
| Physical Address |  |  |
| IP Address |  |  |
| Subnet Mask |  |  |
| Default Gateway |  |  |

| PC-B Network Configuration |  | Points |
| --- | --- | --- |
| Description | You don’t need to configure this. | (2 points) |
| Physical Address |  |  |
| IP Address |  |  |
| Subnet Mask |  |  |
| Default Gateway |  |  |

  - Configure R1.

Configuration tasks for R1 include the following:

| Task | Specification | Points |
| --- | --- | --- |
| Disable DNS lookup |  | (1 point) |
| Router name | R1 | (1 point) |
| Domain name | ccna-lab.com | (1 point) |
| Encrypted privileged exec password | ciscoenpass | (1 point) |
| Console access password | ciscoconpass | (1 point) |
| Telnet access password | ciscovtypass | (1 point) |
| Set the minimum length for passwords | 10 characters | (2 points) |
| Create an administrative user in the local database | Username: admin Password: admin1pass | (2 points) |
| Set login on VTY lines to use local database |  | (1 point) |
| Set VTY lines to accept ssh and telnet connections only |  | (2 points) |
| Encrypt the clear text passwords |  | (1 point) |
| MOTD Banner | Set this with your full name and today’s date (ex) Hwajung Lee 4/21/2017 | (1 point) |
| Interface G0/0 | Set the description Set the Layer 3 IPv4 address Activate Interface | (2 points) |
| Interface G0/1 | Set the description Set the Layer 3 IPv4 address Activate Interface | (2 points) |
| Generate a RSA crypto key | 1024 bits modulus | (2 points) |

  - Configure S1.

Configuration tasks for R1 include the following:

| Task | Specification | Points |
| --- | --- | --- |
| Switch name | S1 | (1 point) |
| Configure Management Interface (SVI) | Set the Layer 3 IPv4 address | (1 point) |
| Encrypted privileged exec password | ciscoenpass | (1 point) |
| Console access password | ciscoconpass | (1 point) |
| Telnet access password | ciscovtypass | (1 point) |

Instructor Sign-off Part 3: ______________________

Points: _________ of 30

- Test and Verify IPv4 End-to-End Connectivity

Ref lab: 8.3.2.7 Lab - Testing Network Connectivity with Ping and Traceroute

Total points: 8

Time: 10 minutes

  - Verify network connectivity.

Use the ping command to test connectivity between all network devices.

Note: If pings to host computers fail, temporarily disable the computer firewall and retest. To disable a Windows 7 firewall, select Start > Control Panel > System and Security > Windows Firewall > Turn Windows Firewall on or off, select Turn off Windows Firewall, and click OK.

Use the following table to methodically verify connectivity with each network device. Take corrective action to establish connectivity if a test fails:

| From | To | IP Address | Ping Results | Points |
| --- | --- | --- | --- | --- |
| PC-A | R1, G0/0 |  | Success or Fail | (1 point) |
| PC-A | R1, G0/1 |  | Success or Fail | (1 point) |
| PC-A | S1 VLAN 1 |  | Success or Fail | (1 point) |
| PC-A | PC-B |  | Success or Fail | (1 point) |
| PC-B | R1, G0/1 |  | Success or Fail | (1 point) |
| PC-B | R1, G0/0 |  | Success or Fail | (1 point) |
| PC-B | S1 VLAN 1 |  | Success or Fail | (1 point) |

In addition to the ping command, what other command is useful in displaying network delay and breaks in the path to the destination? (1 point)

_______________________________________________________________________________________

tracert or traceroute

Instructor Sign-off Part 4: ______________________

Points: _________ of 8

- Use the IOS CLI to Gather Device Information

Ref lab: 11.3.4.6 Lab - Using the CLI to Gather Network Device Information

Total points: 10

Time: 10 minutes

  - Enter the following CLI commands on R1 and see the results. Then, circle each of the commands:

| Command Description | Student Input (command) | Points |
| --- | --- | --- |
| Display a summary of important information about the interfaces on R1. | show ip interface brief | (1 point) |
| Display the IPv4 routing table. | show ip route | (1 point) |
| Display the Layer 2 to Layer 3 mapping of addresses on R1. | show arp | (1 point) |
| Display detailed IPv4 information about interface G0/0 on R1. | show interface g0/0 | (1 point) |
| Display the IPv6 routing table. | show ipv6 route | (1 point) |
| Display a summary of IPv6 interface addresses and status. | show ipv6 interface brief | (1 point) |
| Display information about the devices connected to R1. Information should include Device ID, Local Interface, Hold time, Capability, Platform, and Port ID. | show cdp neighbor | (1 point) |
| Save the current configuration so it will be used the next time the router is started. | copy running-config startup-config | (1 point) |

Instructor Sign-off Part 7: ______________________

Points: _________ of 10

- Router Interface Summary Table

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |