## Task List

### Linux AS Router Setup

- Install Linux

- Determine NIC-to-Interface mappings

- Configure IP address/netmask on each interface

  - eth0: 192.168.1.<1,2>	 / 255.255.255.0

  - eth1: 10.<AS#>0.10.<1,2> / 255.255.0.0

  - eth2: 190.111.50.<11,12,13,14> / 255.255.255.0

- Create default route to  backbone router (190.111.50.55) on eth2

- Enable IP forwarding

- Install Quagga (if not installed)

- Create initial zebra, ospfd, and bgpd config files

- Configure OSPF

  - Router ID: 10.<AS#>0.10.0

  - Networks: 10.<AS#>0.10.0 / 16

  - Originate default route

- Configure BGP

  - Router ID: 190.111.50.<11,12,13,14>

  - Networks: 10.<AS#>0.10.0 / 16

  - iBGP neighbors: 10.<AS#>0.10.<1 or 2,>
                            10.<AS#>0.10.3
                            10.<AS#>0.10.4

  - eBGP neighbors: 192.168.1.<1 or 2>
                            190.111.50.55

### Cisco AS Router Setup

- Initial setup

- Configure IP address/netmask on Ethernet interfaces

  - fa0/0: 190.111.50.<10, 15> / 255.255.255.0

  - fa0/1: 10.<AS#>0.10.<1,2> / 255.255.0.0

- Configure T1 crossover and Serial interface IP address

  - s0/0: 192.168.1.<1,2> / 255.255.255.0

- Create default route to backbone router (190.111.50.55)

- Configure OSPF

  - Router ID: 10.<AS#>0.10.0

  - Networks: 10.<AS#>0.10.0 / 0.0.255.255

  - Originate default route

- Configure BGP

  - Router ID: 190.111.50.<11,12,13,14>

  - Networks: 10.<AS#>0.10.0 / 255.255.0.0

  - iBGP neighbors: 10.<AS#>0.10.<1 or 2,>
                            10.<AS#>0.10.3
                            10.<AS#>0.10.4

  - eBGP neighbors: 192.168.1.<1 or 2>
                             190.111.50.55

### Pod Router Setup

- Initial setup

- Ensure that no default route exists

- Configure IP address/netmask on Ethernet interfaces

  - fa0/0: 190.111.<pod#>.254 / 255.255.255.0

  - fa0/1: 10.<AS#>0.10.<3,4> / 255.255.0.0

- Configure OSPF

  - Networks: 10.<AS#>0.10.0 / 0.0.255.255
                  190.111.<pod#>.0 / 0.0.0.255

- Set OSPF priority to 0 on fa0/1

- Configure BGP

  - Networks: 10.<AS#>0.10.0 / 255.255.0.0
                 190.111.<pod#>.0 / 255.255.255.0

  - iBGP neighbors: 10.<AS#>0.10.1
                            10.<AS#>0.10.2
                            10.<AS#>0.10.<3 or 4>

### Verifying and Testing

- Interface IP address configuration

  - Cisco: show ip int brief

  - Linux: ifconfig

- Routing table

  - Cisco: show ip route

  - Linux: route -n

- OSPF

  - show ip ospf neighbor

- BGP

  - show ip bgp summary

  - show ip bgp neighbors

  - show ip bgp neighbors <neighbor address> advertised routes

  - show ip bgp neighbors <neighbor address> routes

- Ping to and from a router or system within each pod

- Simulate router failure on each AS router

  - Cisco: shutdown router

  - Linux: stop networking service or shutdown interfaces