ITEC 350 Homework 2

- VLSM (Variable Length Subnet Masking)

Given:

Radford Corporation has the following class C Internet License:

193.45.1.0

Subnets:

- AtlantaHQ: 58 hosts

- PerthHQ: 26 hosts

- SydneyHQ: 10 hosts

- CorpusHQ:10 hosts

- WAN1: 2 IP addresses

- WAN2: 2 IP addresses

- WAN3: 2 IP addresses

Questions: Give the answers to the following questions for each subnet including the three WANs.

- How many bits need to be used for a host id?

- How many bits need to be used for a subnet id?

- What is the subnet id?

- What is the subnet mask?

- What is the IP address of THIS subnet?

- What is the IP address of the first usable host id in this subnet?

- What is the IP address of the last usable host id in this subnet?

- What is an address range of this subnet?

- What is the broadcasting address of this subnet in this subnet?