Resetting Routers

Clearing the configuration from a pod router

- Connect the console port of the router to the serial port of a workstation.

  - The red cable at the bottom of the second rack is already connected to the serial port of the lab server (using a console cable and adapter).

- Run Hyperterm

  - Open Start->All programs->Communication->Hyperterm

  - Enter router for the connection name

  - Select COM1 as the serial port

  - Set the port speed to  9600

  - The other options should be set by default to 8 bits, no parity, 1 stop bit, hardware flow control.

- Press <Enter> to bring up the router’s prompt

- Login to the router (each router will have a password that matches its pod or AS#)

  - 350 passwords:  pod1pod1,  pod2pod2, …, podnpodn

  - 451 passwords:  as1as1, as2as2, as3as3

  - If these passwords fail then you will have to do password recovery.

- Enter privileged mode

  - Router>  enable

  - When asked for a password, enter the same password from step 4.

- Erase the startup-configuration

  - Router#  erase start

- Reboot the router

  - Router#  reload

Password Recovery

- Before doing password recovery, try the following password variations:

  - Incorrect case: 	Pod1Pod1,  Pod1pod1, pod1Pod1

  - Short password:  	pod1, Pod1

  - No-numbers:		pod<#>pod<#>,  podxpodx

  - Generic:		password, router

- Make sure Hyperterm is opened and connected to the router

- Enter ROM monitor mode

  - Restart the router by turning the power switch off and on

  - Wait for the first message of the bootup sequence

  - Press Ctrl-Break

  - The router should be at a rommon> prompt or similar

- Set the configuration register to ignore the startup-config (0x2142)

  - rommon> confreg 0x2142

- Restart the router (after restart, no configuration will be loaded).

  - rommon> restart

- Enter privileged mode

  - Router> enable

- Clear the router’s startup-configuration (to keep the existing config, see below)

  - Router# erase start

- Reset the configuration register to normal

  - Router# config t

  - Router (config)# config-register 0x2102

  - Router (config)# end

- Reboot the router

  - Router# reload

  - Select  No when prompted to save the current configuration

Alternate step 7

- Load the startup-config into memory

  - Router# copy start run

- Remove the existing privileged mode password

  - Router# config t

  - Router (config)# no enable secret

  - Router (config)# no enable password

- Remove the existing console password

  - Router (config)# line con 0

  - Rotuer (config-line)# no password

  - Router (config-line)# end

- Save the modified running config

  - Router # copy run start