Packet Tracer – Configuring Multiarea OSPFv2

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | OSPFv2 Area |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 10.1.1.1 | 255.255.255.0 | 1 |
|  | G0/1 | 10.1.2.1 | 255.255.255.0 | 1 |
|  | S0/0/0 | 192.168.10.2 | 255.255.255.252 | 0 |
| R2 | G0/0 | 10.2.1.1 | 255.255.255.0 | 0 |
|  | S0/0/0 | 192.168.10.1 | 255.255.255.252 | 0 |
|  | S0/0/1 | 192.168.10.5 | 255.255.255.252 | 0 |
| R3 | G0/0 | 192.168.2.1 | 255.255.255.0 | 2 |
|  | G0/1 | 192.168.1.1 | 255.255.255.0 | 2 |
|  | S0/0/1 | 192.168.10.6 | 255.255.255.252 | 0 |

Objectives

Part 1: Configure Multiarea OSPFv2

Part 2: Verify and Examine Multiarea OSPFv2

Background

In this activity, you will configure multiarea OSPFv2. The network is already connected and interfaces are configured with IPv4 addressing. Your job is to enable multiarea OSPFv2, verify connectivity, and examine the operation of multiarea OSPFv2.

Configure OSPFv2

Configure OSPFv2 on R1.

Configure OSPFv2 on R1 with a process ID of 1 and a router ID of 1.1.1.1.

Advertise each directly connected network in OSPFv2 on R1.

Configure each network in OSPFv2 assigning areas according to the Addressing Table.

R1(config-router)# network 10.1.1.0 0.0.0.255 area 1

R1(config-router)# network 10.1.2.0 0.0.0.255 area 1

R1(config-router)# network 192.168.10.0 0.0.0.3 area 0

Configure OSPFv2 on R2 and R3.

Repeat the steps above for R2 and R3 using a router ID of 2.2.2.2 and 3.3.3.3, respectively.

Verify and Examine Multiarea OSPFv2

Verify connectivity to each of the OSPFv2 areas.

From R1, ping each of the following remote devices in area 0 and area 2: 192.168.1.2, 192.168.2.2, and 10.2.1.2.

Use show commands to examine the current OSPFv2 operations.

Use the following commands to gather information about your OSPFv2 multiarea implementation.

show ip protocols

show ip route

show ip ospf database

show ip ospf interface

show ip ospf neighbor

Reflection Questions

Which router(s) are internal routers? _____________________________________________

Which router(s) are backbone routers? _____________________________________________

Which router(s) are area border routers? _____________________________________________

Which router(s) are autonomous system routers? _____________________________________________

Which routers are generating Type 1 LSAs? _____________________________________________

Which routers are generating Type 2 LSAs? _____________________________________________

Which routers are generating Type 3 LSAs? _____________________________________________

Which routers are generating Type 4 and 5 LSAs? _____________________________________________

How many inter area routes does each router have? _____________________________________________

Why would there usually be an ASBR in this type of network? ______________________________________

Suggested Scoring Rubric

Packet Tracer scores 80 points. Each of the Reflection Questions is worth 2 points.