# Classification of Routing Protocols within Wireless Sensor Networks: A Survey

Cody S. Moler

Abstract—Wireless sensor networks (WSNs) provide an abundance of applications that constitute a seemingly limitless variety of scenarios. Because these sensor networks are constructed around a distributed nature and dynamic topology, the routing protocols designed for them must meet many specific requirements. Along with this, the appropriateness of a particular routing protocol will depend upon the situation it is being used for and the capabilities of the nodes within the network itself. This paper presents the requirements and constraints of routing protocols in wireless sensor networks along with a review of the current protocols in production that will consist of their classifications and comparisons. Ultimately, this will reveal the prominent features that will need to be taken into consideration in the design and evaluation of future routing protocols for sensor networks.

Index Terms—routing protocol, wireless sensor network

# INTRODUCTION

A

WSN is a collection of low-power multi-functional sensor nodes, ranging in number from hundreds to thousands, that may be mobile or stationary and are located either precisely or randomly on a dynamically changing environment. The fundamental components of a node are a sensor unit, an ADC (Analog to Digital Converter), a CPU (Central processing unit), a power unit and a communication unit [1]. These networks have come forth as a promising tool for monitoring the physical world from their capabilities to sense, process and communicate. Surfacing from the military field with the motivation of applications such as battlefield surveillance, these sensor networks are used today in many industrial and civilian application areas, such as healthcare, environment and habitat monitoring, home automation, machine health monitoring, traffic control and so on.

Some of the advantages of sensor nodes are that they are made small, consume extremely low energy, are operated in high volumetric densities, and can be autonomous and adaptive to the environment [2]. However, some of these benefits are also the cause of their drawbacks. Because wireless sensor nodes are usually considerably small electronic devices, they can only be equipped with a limited power source [3]. Also, each sensor node possesses a certain area of coverage in which it can accurately and reliably report the condition it is observing. Therefore, because of these hindrances, a large number of sensor nodes are distributed over a chosen location for collecting information (pressure, humidity, temperature, motion detection, etc.). These sensor nodes can communicate with each other through the distribution of information either directly or through other intermediate nodes, thus forming a network. In this way, each node in a WSN acts as a router inside the network [4].

A benefit of WSNs is that new nodes can be added to the network at any time. Because of this and the unpredictability of some of the environments these WSNs exist in, the topology of the sensor network can change very frequently. These factors cause identifying the nodes to be an issue; one that must be attended too before moving forward with the designation of the network. This issue is addressed later in this paper.

Once the nodes are identified, routing protocols are responsible for constructing and maintaining routes between distant nodes. In this sense, each router has a prior knowledge of only the networks directly attached to it. The protocol is what shares this information among immediate neighbors and later throughout the network. Ultimately, each node will gain knowledge of the topology of the network. The diverse ways in which routing protocols operate are what make them appropriate for certain implementations. Within this field and related literature, there are many proposals featuring routing algorithms and protocols in wireless sensor networks. This paper aims at describing the more relevant examples and the attributes that they are made up of in order to promote a further understanding of the different routing techniques that can be applied to WSNs.

The rest of the paper is structured as follows. Section II will describe the components of routing protocols in WSNs to build a foundation on which the later shown related works can be applied. Part II-A briefly shows the basic communication paradigms that WSNs act in accordance with while part II-B describes the chief design constraints and requirements that proposed routing protocols must tackle. Part II-C presents the most favored classification schemes for routing protocols in WSNs and part II-D provides an outline of the multiple optimization procedures accepted by these routing protocols. Section III provides insight to the existing research applications given by other researchers in this topic. Parts III-A through III-D gives examples of these routing protocols and their applications within the earlier explained optimization techniques. Part III-E provides a comparison of these protocols, including their strengths and weaknesses. This is shown in hope that a categorization of these protocols will provide a structure in choosing the optimal routing protocol for a given situation. Section IV imparts a cursory look at potential future research applications for WSNs. Finally, Section V draws the main conclusions of this work.

# Wireless Sensor Network Components

## Algorithm Paradigms for WSNs

The ability to perform cooperative tasks as well as sending and receiving data packets would not be possible without the specifically designed algorithms for these networks. The following will briefly describe three of these different classes of algorithms for WSNs [5].

### Centralized Algorithms

These algorithms execute in a central node that holds the knowledge of the entire network. They are not very common in WSNs because of the excessive cost of transmitting the data to make the node understand the status of the whole network.

### Distributed Algorithms

This paradigm is an algorithm that executes on different sensor nodes within the network, in which their communication is supported by message-passing, similar to a typical computational model.

### Localized Algorithms

Within these algorithms, the nodes use restricted data gained from the local area instead of a global network knowledge like other algorithm paradigms. From this local information, the algorithm is executed in a single node.

From this, it is clear to see that it is important to take into consideration the algorithm paradigm when choosing the routing protocol to employ in a WSN. Each has its own advantages, such as centralized algorithms combining messages from dissimilar sources for simultaneous communication to the central node. With distributed algorithms, one can efficiently support transmissions between any two pairs of nodes. Their hindrances must also be applied as well. For example, local based algorithms depend on geographic coordinates, like GPS, making solutions for them a more resource expensive process [6].

  - Design Constraints for Routing in WSNs

As stated earlier, there are a large number of situations and environments in which WSNs can be utilized. In any of these scenarios the architecture and design of these WSNs can differ. One can consist of heterogenous and mobile sensor nodes, or even apply a topology as simple as a star topology. In essence, the scale and density of a network varies depending on the application. To attain these diverse needs, the following shows some significant design issues a sensor network must attest too in order to be considered functional.

### Fault Tolerance

Fault tolerance is the ability to sustain sensor network functionalities without interruption from unpredictable sensor node failures. These sensors may unpredictably stop operating due to environmental causes or from excessive battery consumption [7]. The failure of sensor nodes should not affect the overall functionality of the WSN, and it is the job of the routing protocols to cope with this eventuality. In this case, when an operational node fails, an alternative route could be discovered and put into use [6].

### Node Deployment

An application dependent issue is the topological deployment of nodes, which affects the performance of the routing protocol. The distribution of nodes is either deterministic or self-organizing. In the case of deterministic distribution, the sensor nodes are manually placed, and data is routed through pre-determined paths. However, in self-organizing systems the sensor nodes are spread over the area of interest randomly, which creates an infrastructure in an ad hoc manner [2,7-8]. Inside of this infrastructure, the position of the sink, or base-station, is likewise crucial in the case of energy-efficiency and performance.

### Power Consumption and Energy Efficiency

Considerations of energy efficiency should always be applied during the creation of a WSN infrastructure and the process of setting up node routes. The sensor nodes in WSNs have a limited supply of energy and they use it for many important processes, such as computation, communication and sensing. In the case of some routing protocols, sensor nodes will take part in data fusion, which only expends more energy. It should also be noted that the battery replacement in these sensors can be infeasible since most of the them are randomly placed, and under some circumstances even unreachable.

Because the transmission power of a wireless radio on these nodes is proportional to distance squared, or even higher in the presence of obstacles, multi-hop routing will consume less energy than direct communication [7]. However, this method requires some route management overhead. Direct routing would really only be used effectively if all the nodes were within close proximity to the sink [9]. A majority of the time the sensors are scattered randomly over an area of interest and multi-hop routing becomes unavoidable [8].

### Scalability

Within a WSN the number of sensor nodes deployed may be in the order of hundreds to thousands. Routing protocols must be workable with this substantial number of nodes. They must also possess the ability to function properly in the event that more nodes are later added to the network.

### Mobility Adaptability or Network Dynamicity

Because the nodes in WSNs can be static or dynamic, dynamicity of the network is a challenging issue. Routing messages from or to moving nodes is more of a challenge since route stability becomes a critical optimization factor [8]. In most cases a routing protocol will assume that the sensor nodes and base stations are static, but in some situations mobility will be needed and the routing protocols must be able to render appropriate support for these movements. For example, in target/tracking applications, the event is dynamic, whereas forest monitoring for early fire prevention is a static event [2].

This issue also applies to when an event is reported in the network. Static event monitoring allows the WSN to be in a reactive mode and will only generate traffic when it is reporting. Dynamic events will require periodic reporting which will therefore generate significant traffic to be routed to the sink [8].

## Classification of Routing Protocols in WSNs

Once their procedures have been considered, routing protocols can be roughly classified according to the following criteria.

### Flat Scheme or Hierarchy Role of Nodes

In a flat scheme, every sensor node possesses the same role in the routing procedures. In a different regard, the hierarchical routing protocols classify sensor nodes according to their functionalities [10]. Afterwards, the network is divided into clusters. A cluster head is then selected in the cluster to coordinate its activities and to communicate with sensor nodes outside of its own cluster. As stated in Section B, the differentiation of nodes can be static or dynamic [6].

### Data Delivery Model

Depending on the application, the gathering of data and network interaction can be accomplished in a couple diverse ways. Data delivery models will determine the flow of information between the sensor nodes and the sink. Depending on the application of the WSN, the data delivery model can be any of the following: Continuous, Event-driven, Query-driven or Hybrid [10]. In the continuous model, the sensor nodes send data to the sink periodically. In contrast, within the event-driven model the sensors report information only if an event of interest occurs. The query-driven model is similar in that the transmission of data is triggered when a query is generated by the sink [7]. Lastly, some networks can be applied with a hybrid model using a combination of the previous models.

The routing protocol is highly influenced by the data delivery model and especially affects the minimization of energy consumption and route stability within the network [8]. For instance, consider an application of a WSN for habitat monitoring, and within this network data is continuously transmitted to the sink. In this case a hierarchical routing protocol would be the most efficient choice due to the fact that the WSN would generate a significant amount of redundant data that could be aggregated on route to the sink, therefore reducing traffic and saving an abundance of energy.

## Optimization Techniques for Routing in WSNs

The showcased constraints and specific characteristics of WSNs have caused the need for particular requirements for routing protocols in this field of study. The following shows some of the optimization techniques that researchers have found particularly important to understand and explore regarding these routing protocols.

### Data-centric and Attributed-based

In data-centric routing, the sink sends queries to specific regions and waits for data from the sensor nodes located in those regions. This type of routing also follows an attributed-value scheme. Because the data is being requested through queries, attributed-based naming is necessary to specify the properties of data [8]. With this, the queries inform about the required data. A characteristic to note in these schemes is that the content of the data messages is analyzed in each hop to make decisions about routing [6].

### Location-based

Within sensor networks, most routing protocols require location information for the sensor nodes. Usually location information is needed in order to calculate the distance between nodes in order to estimate energy consumption. This is partly because there is no addressing scheme for WSNs like regular IP-addresses and also because they are spatially deployed in a region [8]. When this technique is executed, a node decides the transmission route according to the location of the final destination and the positions of other nodes in the network.

### Multipath Communication

With multipath communication, nodes utilize multiple paths from their origin to a destination within the WSN. Because these communications are intended to enhance the reliability and performance within the network, these paths should not share a link [6]. There are two ways to create multipath communications. In the first method one path is established as the active communication routing path, while the other paths are stored for future demands, an example of which would be if the current active path is broken. With the second method it is possible to distribute the traffic among the multiple paths for greater utilization.

### Quality of Service

While not particularly required for every use of a WSN, some applications or their functionalities may need the assurance of Quality of Service (QoS). For instance, some of these functionalities would include effective sample rate, delay bounded and temporary precision [6]. Not all routing protocols can satisfy this demand because it may in fact be in direct opposition of the protocols principles. For example, a particular routing protocol may very well be designed to increase the network’s lifetime while an application could require an effective sample rate that forces periodic transmissions, which causes periodic energy consumptions. Therefore, this technique is more of a useful niche than the others.

### Energy Efficiency

As stated earlier, a particularly noteworthy constraint with a wireless sensor network is energy consumption. It is the focus of many researchers to make their routing protocols energy efficient in order to mitigate this limitation. Within a WSN there are multiple routes that can be used to communicate among the nodes and the sink. The purpose of energy-aware algorithms is to pick the routes that can be expected to augment the network lifetime.

# Related Research

So far, this paper has provided the foundation needed to recognize the functions of routing protocols within wireless sensor networks. This section provides the main focus of this survey by showing the existing fields of research on routing protocols. To give a more thorough understanding of this subject, the routing protocols given below have been categorized by their applications of the optimization techniques listed in the previous sections. Please note that there are many protocols developed in this field that are not listed in this paper. The ones showcased below are the more prominent protocols that have, at this point in time, received the most attention and research.

## Data-centric and Attributed-based Routing Protocols

In this category, the following protocols have gained the most exposure in the research field:

### SPIN

SPIN (Sensor Protocols for Information via Negotiation) is one of the first routing protocols developed for WSNs that pursues a data-centric routing mechanism. The authors in [11] present this adaptive protocol which is made to efficiently disseminate information among sensors in an energy-constrained WSN. The idea behind SPIN is to name the data using high-level descriptors or meta-data. Before transmission, meta-data are exchanged among the sensors via a data advertisement mechanism, which is the key feature of SPIN [8]. The reason these researchers chose to use meta-data negotiations is to eliminate the transmission of redundant data in the network. Once each node receives new data, it advertises the data to its neighbors and any interested neighbors that do not have the data, and lastly retrieves the data by sending a request message. This process is very similar to the commonly known three-way handshake.

One of the biggest advantages of SPIN is that nodes can base their communication decisions on both application-specific knowledge of the data and on knowledge of the resources that are available to them. From this, the sensors can efficiently distribute data if they possess a limited energy supply [6]. Another benefit is that topological changes are localized because each node needs to know only its single-hop neighbors. However, this protocol also possesses some drawbacks. For instance, SPIN’s data advertisement mechanism cannot guarantee full data delivery. An example would be if the nodes that are interested in the data are distant from the source node and the nodes between the source and destination are not interested in that particular data, then the data will not be delivered to the destination in the first place [8]. From this, one could derive that SPIN would not be an appropriate choice for applications such as intrusion detection because they would require reliable delivery of data packets over regular intervals.

### Directed Diffusion

Directed Diffusion (DD) is another important data-centric routing protocol being created and researched by [12]. The goal of this protocol is to diffuse data through sensor nodes by implementing a naming scheme for the data. This is in order to remove unnecessary operations of network layer routing in order to save energy and minimize the number of transmissions. The scheme these researchers have implemented is the naming of data by attribute-value pairs.

There are four elements that make up this protocol: interests, data messages, gradients and reinforcements [13]. A node which wants the data will generate a request where an interest is specified according to the attribute-value based scheme defined by the application. For each application task, the sink injects an interest in the network. The nodes then update an internal interest cache with the interest messages they have received, while also keeping a data cache for recent data messages [8]. These are stored to assist in determining the data rate for transmission. Once the message has been received, the nodes establish a gradient, otherwise known as a reply link, to the originator of the interest. The gradient is characterized by the data rate, duration and expiration time. Furthermore, the node begins to collect the intended data by activating its sensors.

When a node receives an interest message it establishes multiple gradients, or first hop in a route, to the sink. It is imperative that the node identifies the optimum gradient for the most effective communication. To do this both positive and negative reinforcements are implemented. This algorithm uses exploratory and data gradients. The first are intended for route set-up and repair while the later are used for the transmission of real data [12].

As stated before, the main advantage of Directed Diffusion is saving energy by selecting optimal paths by caching and processing data in the network [2]. As with all protocols however, it has some drawbacks as well. For starters, to create data aggregation the protocol requires a time synchronization technique, which is not a simple matter to achieve in WSNs. Additionally, the protocol requires much overhead regarding the recording of information, which only adds to the cost of a sensor node [13].

### COUGAR

With COUGAR [14], the network is viewed as a huge distributed database system where some nodes containing information are temporarily unreachable. The network behaves as a data warehouse because of nodes storing historic values. In this architecture, nodes select a leader node to perform aggregation and transmit the data to the sink. The sink is responsible for creating a query plan, which consists of the necessary information about the data flow and in-network computation for the incoming query, in which it afterwards sends to the desired nodes [8].

This routing protocol has an advantage in that it provides higher level services because of the queries that can be issued from a sink. It drawbacks, however, are similar to those of Directed Diffusion. Introducing an additional query layer on each sensor node brings increased overhead to all nodes, which impacts energy consumption and storage. Secondly, the in-net-word data computation will require synchronization between the nodes [14]. Also, this protocol cannot be directly applied to a WSN and instead must be modified on the base of the application.

## Geographical/Location-based Routing Protocols

These protocols make use of the location information to make routing techniques more efficient. The following protocol is currently the most accepted and well researched in this category.

### GAF

GAF (Geographic Adaptive Fidelity) [15] is an energy-aware location-based routing algorithm that conserves energy by turning off unnecessary nodes in the network without affecting the level of routing fidelity. This protocols aim is to optimize the performance of a WSN by identifying equivalent nodes for the forwarding of packets. In GAF, two nodes are considered equivalent when they maintain the same set of neighbor nodes, which indicates that they can belong to the same communication routes. From this, the applications source and destination are excluded in this characterization [8].

In order to identify equivalent nodes their positions must be established. To do this each node uses its GPS-indicated location to associate itself with a point in a virtual grid. The virtual grid is constructed from the cells whose size allows to state that all nodes within one cell can directly communicate with the nodes belonging to adjacent cells and vice versa [6]. From this, the nodes in a cell are equivalent in terms of the cost of packet routing. The equivalence is utilized by keeping some nodes located in a specific grid area in a sleeping state to save energy, thus increasing the network lifetime as the number of nodes increases within the WSN [2].

## Hierarchical Routing Protocols

The leading objective of hierarchical routing is to reduce energy consumption by classifying nodes into clusters, and perhaps also performing data aggregation and fusion to decrease the number of messages being transmitted to the sink [8]. The process of cluster formation is usually based on the sensor’s energy reserves and their proximity to the sink [16]. This section will explore the, currently, most influential hierarchical routing protocols.

### LEACH

LEACH (Low-energy adaptive clustering hierarchy) [9] is one of the first and most popular hierarchical routing algorithms for WSNs. The proposal is to form clusters of the sensor nodes based on the signal strength that is received and to use local cluster heads as routers to the sink [8]. This will support the conservation of energy because the transmissions will only be done by these cluster heads rather than all of the sensor nodes within the network. The role of the cluster head is periodically and randomly transferred among the nodes in the network to balance energy dissipation of the nodes [9].

The execution of LEACH is based on rounds. Each round a cluster head is elected, and it establishes a schedule for the transmissions in its cluster [6]. The schedule it produces allows nodes to switch off their interfaces if they are not to be employed in that round. The benefit of the data aggregation performed by the cluster head is the removal of redundancy.

The biggest advantage of LEACH that has not already been mentioned is that it is completely distributed and requires no global knowledge of the network [9]. However, this protocol uses single-hop routing, wherein each node has the ability to transmit directly to the cluster-head and the sink. This makes LEACH not applicable to WSNs deployed in very large areas. Also, there is very high overhead because of the dynamic clustering involved. Every cluster-head change, advertisement, etc. has the ability to lessen the advantage of energy consumption [8].

### PEGASIS

PEGASIS (Power-Efficient Gathering in Sensor Information Systems) [17] is considered an improvement of the LEACH protocol. Rather than classifying nodes in clusters, the protocol forms chains of the sensor nodes. Following this structure, each node transmits and receives from only one node of its neighbors and only one node is selected from the chain to transmit to the sink [8]. Once the chain is created, the data gathered moves from node to node, aggregated and eventually sent to the sink.

The difference from LEACH is that PEGASIS uses multi-hop routing by forming chains and selecting only one node from transmission to the sink instead of using multiple nodes as cluster-heads. It outperforms LEACH by removing the overhead of dynamic cluster formation and limits the number of transmissions [2]. However, PEGASIS still requires dynamic topology adjustment because of the sensor’s energy not being tracked. In this protocol every sensor node needs to know its neighbor’s status so that it understands where to route the data [17]. This topology adjustment will most likely introduce significant overhead, especially in high traffic WSNs.

### TEEN and APTEEN

TEEN (Threshold Sensitive Energy Efficient Sensor Network Protocol) [18] is a hierarchical protocol designed to be responsive to abrupt changes in the sensed properties, such as temperature, pressure, etc. This function is important for time-critical applications, wherein the WSN operates in a reactive mode. With this protocol the architecture of the network is based on a hierarchical grouping where nearby nodes form clusters and this process continues into multiple levels of cluster heads until the sink is reached [8].

Once the clusters are formed, the cluster head broadcasts a hard and soft threshold for sensed properties to the nodes. A hard threshold is the minimum possible value of a property to trigger a sensor node to switch on its transmitter and transmit data to the cluster head [18]. This allows the sensor nodes to transmit only when the property is in the range of interest, which significantly reduces the number of transmissions. The function goes as follows: The nodes transmit data if the current value of the sensed property is greater than the hard threshold and when it differs from the sensed value by an amount equal to or greater than the soft threshold [6,18].

This strategy aims to reduce energy consumption during the transmission of messages, which it does a good enough job of. However, TEEN is not very useful for applications where the periodic transmission of data is required because the nodes will never communicate if the thresholds are not reached. In this case the user will never get any data from the WSN and will also never know if any of the nodes fail [6].

APTEEN (Adaptive Threshold Sensitive Energy Efficient Sensor Network Protocol) [19] is an extension of TEEN and aims to prevent TEEN’s drawbacks by not only reacting to critical events but also capturing periodic data collections. Other than this key difference, the architecture is the same as in TEEN. While removing one of TEENs main drawbacks, doing so makes APTEEN sacrifice some of its network lifetime [8]. Also, both of these protocols still have the disadvantage of large overhead and complexity because of the formation of clusters on multiple levels [18,19].

## Multipath Routing Protocols

Within multipath routing protocols, a source knows multiple routes to a destination. With this approach, the routes can be used simultaneously, or with one active and the rest as backups for future demands. Within this section, a routing protocol that is currently known to be the most popular among multipath protocols for WSNs will be shown.

### SAR

SAR (Sequential Assignment Routing) was proposed in [20] and is one of the first protocols designed for WSNs that includes the notion of QoS in its routing decisions. The goal of this protocol is to achieve energy efficiency and fault tolerance [8]. The routing decisions in SAR are dependent on three factors: energy resources, QoS on each path, and the priority level of each packet. A multi-path approach and localized path restoration schemes are utilized to avoid single route failure [20]. For the protocol to create multiple paths from a source node, a tree that is rooted at the source node to the destination nodes is created. The paths of the tree are constructed while avoiding sensor nodes where low energy or QoS are guaranteed. Once this process is finished, each sensor node will be a part of a multi-path tree [21].

By using the created trees, multiple paths from the sink to the sensors are formed within the WSN. One of these paths is selected according to the energy resources and QoS on the path [8]. The procedure of failure recovery is accomplished by enforcing routing table consistency between upstream and downstream nodes on each path. An automatic path restoration procedure is called locally in the case of any local failure [20]. The biggest drawback of SAR is the overhead of maintaining the tables and states at each sensor node, especially when the number of nodes is substantial. However, the benefits of fault-tolerance and easy recovery could make up for the losses.

## Comparisons

All of the protocols showcased have differences and similarities amongst themselves and within their categories. This section will feature the comparison of these techniques and protocols.

### Classification Techniques

Regarding the data-centric and attributed-based protocols, these algorithms become relevant when the data sensed by the nodes are not usually interesting to the rest of the nodes in the WSN. Because of this they usually greatly reduce network overhead. Truly, the decision of which protocol to select mainly depends upon the data delivery model that the application forces [6]. For instance, SPIN is the most suitable data-centric protocol to choose if the communication should be triggered by events. However, SPIN cannot guarantee data delivery, so when reliable data delivery is required Directed Diffusion might be the best choice in this category.

Hierarchical and geographic routing protocols are both considered scalable solutions [8]. A hierarchical structure requires the coordination of nodes from the transmission of messages. The use of cluster-based structures makes up for this drawback in dense networks, but does not hold up in small networks [2]. Geographic protocols exhibit a similar behavior. In an extended network with many nodes, the exchange of messages to discover neighbor locations is usually intensive. However, it becomes negligible compared to the reduction of transmissions that geographic protocols achieve. Nevertheless, the topology of the network in both of these categories must remain stable and the cluster structure and geographic information must be frequently updated, leading to supplementary costs [6].

Compared to the other classifications, the main disadvantage of multipath routing protocols lies on the cost of maintaining the paths in the WSN. Automatically increasing network overhead, this cost also uses memory resources. Multipath protocols should not be used for WSNs that have incredibly strict power constraints. On the other hand, they almost become necessary when applications have a strong need for reliability.

### Routing Protocols

SPIN, DD and COUGAR use meta-data, whereas the other protocols don’t. In DD the sink sends queries to the sensor nodes by flooding the network, but in SPIN the nodes advertise data availability so that interested nodes can query that data [2]. Also, within DD each node can communicate with its neighbors, so it does not maintain a global network topology, whereas SPIN does.

LEACH, PEGASIS, TEEN and APTEEN all have similar features and architectures to some extent. Of the four, PEGASIS is the only chain-based protocol, while the others are cluster-based. Simulations provided in [19] show that the performance of APTEEN lies between TEEN and LEACH regarding energy consumption and network longevity. As stated earlier, TEEN only transmits time-critical data, while APTEEN performs periodic data transmissions. This also shows that APTEEN is sometimes superior to LEACH because it transmits data based on a threshold value and LEACH does so continuously. PEGASIS avoids overhead from clustering like the others, but it introduces excessive delay for distant nodes on the chain [2,17]. Because of this, the single leader can become a bottleneck. However, simulations show that PEGASIS increases network lifetime two-fold compared to LEACH [17].

Because wireless sensor networks are application specific, it is impossible to say which particular protocol is better than another. One can only look at the functions and the drawbacks from those functions and choose which protocol suits the applications needs the most. In Table I shown below, a classification and comparison of all of the protocols mentioned in this paper is demontrated. Each protocol is described with the characteristics and techniques it provides.

# Future Research

Although many breakthroughs have been made regarding the routing problem in the field of WSNs, it still poses many challenges that will need to be tackled in the future. The performance of the protocols shown in this paper are promising in terms of energy efficiency, but the energy constraints of WSNs will always be a focal point in this field. Also, further research should be addressed to issues such as QoS posed by video and imaging sensors and real-time applications. Future QoS routing protocols should hope to ensure guaranteed bandwidth throughout the duration of a connection as well as providing the most energy efficient path.

One of the other most interesting issues for routing protocols in WSNs is the consideration of node mobility. At this point in time most current protocols assume that the sensor

TABLE I

Classification and Comparison of Routing Protocols in Wireless Sensor Networks

| Routing Protocol | Classification | Power Usage | Mobility | Data Aggregation | QoS | Scalability | Multipath | Query-based | Overhead | Delivery Model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SPIN | Data-centric/Flat | Limited | Possible | Yes | No | Limited | Yes | Yes | Low | Event driven |
| Directed Diffusion | Data-centric/Flat | Limited | Limited | Yes | No | Limited | Yes | Yes | Low | Demand driven |
| COUGAR | Data-centric/Flat | Limited | No | Yes | No | Limited | No | Yes | High | Query driven |
| GAF | Location | Limited | Limited | No | No | Good | No | No | Moderate | Virtual grid |
| LEACH | Hierarchical | Max | Fixed Sink | Yes | No | Good | No | No | High | Cluster-head |
| PEGASIS | Hierarchical | Max | Fixed Sink | No | No | Good | No | No | Low | Chains-based |
| TEEN & APTEEN | Hierarchical | Max | Fixed Sink | Yes | No | Good | No | No | High | Active threshold |
| SAR | QoS/ Multipath | N/A | No | Yes | Yes | Limited | Yes | Yes | High | Continuously |

nodes and the sink are stationary. However, there are many situations, such as battlefield environments or in-flight scenarios, where the sink and sensors must have the ability to be mobile. In this case, there would be frequent updates of the positions of the nodes and sink, which would increase the propagation of information within the network, therefore excessively draining the energy of the nodes. Entirely new routing algorithms will be needed in order to manage the massive overhead of mobility and topology updates in such an energy constrained scenario. Fortunately, researchers are already approaching this issue such as the authors of [22].

Finally, secure routing is another hot topic for WSNs. Current routing protocols in this field have optimizations for the nodes limited capabilities and the application specific nature of the networks, but hardly any consider security. The protocols shown in this paper were not designed with security as a goal, but it is imperative to analyze their security properties. It will be difficult to design secure routing protocols for sensor networks, especially because of in-network aggregation [21], but once researchers find a method WSNs will be pushed into even further significance.

# Conclusion

Because of their many applications, wireless sensor networks have been an incredible resource for many institutions, and will only continue to grow as a field. Still being a somewhat new topic of study, there has been a need for the creation of many new components, such as routing protocols that are fundamentally similar and at the same time different from traditional routing protocols used for regular networks. For each of the many different applications of WSNs there is a necessity to design a practical and efficient protocol to fit their needs.

Firstly, this paper builds a foundation of the routing protocols by providing their many components, such as the different algorithm paradigms, design constraints, classifications, and optimization techniques. With this, a secure understanding of the fundamental design process of these protocols can be achieved. Furthermore, this paper showcases some of the most prominently related routing protocols that have been created and researched within this field. Those protocols being: SPIN, DD, COUGAR, GAF, LEACH, PEGASIS, TEEN, APTEEN and SAR. Because sensor networks are application specific, no protocol can be said to be better than the other. In order to give some assistance in this regard, a classification and comparison of the techniques of these protocols has been provided. Lastly, this paper gives some insight into some of the future research challenges and directions of wireless sensor networks.

- References

- F. Lewis, “Wireless Sensor Networks”, Automation and Robotics Research Institute, The University of Texas at Arlington: Ft. Worth, Texas, USA, 2004, pp. 1-18.

- D. Bhattacharyya, T. Kim and S. Pal, "A Comparative Study of Wireless Sensor Networks and Their Routing Protocols", Sensors, vol. 10, no. 12, pp. 10506-10523, 2010.

- C. Schurgers and M. Srivastava, “Energy efficient routing in wireless sensor networks”, The MILCOM Proceedings on Communications for Network-Centric Operations: Creating the Information Force, McLean, VA, USA, 2001.

- R. Shah and J. Rabaey, “Energy aware routing for low energy ad hoc sensor networks”, Proceedings of the IEEE Wireless Communications and Networking Conference (WCNC), Orlando, FL, USA, March 2002.

- A. Boukerche, E. Nakamura and A. Loureiro, “Algorithms for Wireless Sensor Networks”, Algorithms and Protocols for Wireless Sensor Networks, John Wiley & Sons: Hoboken, NJ, USA, 2009.

- L. García Villalba, A. Sandoval Orozco, A. Triviño Cabrera and C. Barenco Abbas, "Routing Protocols in Wireless Sensor Networks", Sensors, vol. 9, no. 11, pp. 8399-8421, 2009.

- R. Biradar, V. Patil, S. Sawant and R. Mudholkar, “Classification and Comparison of Routing Protocols in Wireless Sensor Networks”, Special Issue on Ubiquitous Computing Security Systems, vol. 4, no. 2, pp. 704-711, 2009.

- K. Akkaya and M. Younis, "A survey on routing protocols for wireless sensor networks", Ad Hoc Networks, vol. 3, no. 3, pp. 325-349, 2005.

- W. Heinzelman, A. Chandrakasan and H. Balakrishnan, "Energy-efficient communication protocol for wireless microsensor networks", Proceedings of the 33rd Annual Hawaii International Conference on System Sciences, 2000.

- S. Tilak, N. Abu-Ghazaleh and W. Heinzelman, "A taxonomy of wireless micro-sensor network models", ACM SIGMOBILE Mobile Computing and Communications Review, vol. 6, no. 2, pp. 28-36, 2002.

- J. Kulik, W. Heinzelman and H. Balakrishnan, "Negotiation-based protocols for disseminating information in wireless sensor networks", Wireless Networks, vol. 8, no. 23, pp. 169-185, 2002

- C. Intanagonwiwat, R. Govindan and D. Estrin, "Directed diffusion: a scalable and robust communication paradigm for sensor networks", Proceedings of the 6th annual international conference on Mobile computing and networking - MobiCom '00, pp. 56-67, 2000.

- Q. Jiang and D. Manivannan, “Routing protocols for sensor networks”, Proceedings of Conference on Consumer Communications and Networking, pp. 93-98, 2004.

- Y. Yao and J. Gehrke, "The cougar approach to in-network query processing in sensor networks", ACM SIGMOD Record, vol. 31, no. 3, pp. 9-18, 2002.

- Xu, J. Heidemann and D. Estrin, "Geography-informed energy conservation for Ad Hoc routing", Proceedings of the 7th annual international conference on Mobile computing and networking - MobiCom '01, pp. 70-84, 2001.

- C. Lin and M. Gerla, "Adaptive clustering for mobile wireless networks", IEEE Journal on Selected Areas in Communications, vol. 15, no. 7, pp. 1265-1275, 1997.

- S. Lindsey and C. Raghavendra, "PEGASIS: Power-efficient gathering in sensor information systems", Proceedings of the IEEE Aerospace Conference, 2002.

- A. Manjeshwar and D. Agrawal, "TEEN: a routing protocol for enhanced efficiency in wireless sensor networks", Proceedings of 15th International Parallel and Distributed Processing Symposium, 2001.

- A. Manjeshwar and D. Agrawal, "APTEEN: a hybrid protocol for efficient routing and comprehensive information retrieval in wireless sensor networks", Proceedings of the 2nd International Workshop on Parallel and Distributed Computing Issues in Wireless Networks and Mobile computing, 2002.

- K. Sohrabi, J. Gao, V. Ailawadhi and G. Pottie, "Protocols for self-organization of a wireless sensor network", IEEE Personal Communications, vol. 7, no. 5, pp. 16-27, 2000.

- J. Al-Karaki and A. Kamal, "Routing techniques in wireless sensor networks: a survey", IEEE Wireless Communications, vol. 11, no. 6, pp. 6-28, 2004.

- S. Awwad, C. Ng, N. Noordin and M. Rasid, "Cluster Based Routing Protocol for Mobile Nodes in Wireless Sensor Network", Wireless Personal Communications, vol. 61, no. 2, pp. 251-281, 2010.

C. S. Moler is studying in the Department of Information Technology, Radford University, Radford, VA 24142 USA (e-mail: cmoler@radford.edu).